| Hodgdon Powder Company Cartridge Load Recipe Report - 10/30/2003 | 7.5 x 55mm Swiss. Load Type:; Rifle. |
| --- | --- |
| Comments: This is everything we have on the 7.5 Mike |  |

| Cartridge Information |  |
| --- | --- |
| Case:; Norma; Barrel Length:; 31". Twist:; 1:10.6"; Trim Length:; 2.175". Primer:; WINCHESTER LR. |  |

| 7.5 x 55mm Swiss Cartridge Load Data |  |  |  | Starting Loads |  |  | Maximum Loads |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bullet Weight (Gr.) | Powder | Bullet Diam. | C.O.L. | Grs. | Vel. (ft/s) | Pressure | Grs. | Vel. (ft/s) | Pressure |
| 125 GR. SIE SP | H4831 | .308" | 2.800" | 49.0 | 2639 |  | 52.0 | 2811 |  |
| 125 GR. SIE SP | H4350 | .308" | 2.800" | 47.0 | 2565 |  | 51.0 | 2839 |  |
| 125 GR. SIE SP | H414 | .308" | 2.800" | 46.0 | 2652 |  | 48.0 | 2829 |  |
| 125 GR. SIE SP | H380 | .308" | 2.800" | 44.0 | 2661 |  | 46.0 | 2797 |  |
| 125 GR. SIE SP | BL-C(2) | .308" | 2.800" | 41.0 | 2614 |  | 43.0 | 2739 |  |
| 125 GR. SIE SP | H335 | .308" | 2.800" | 40.0 | 2587 |  | 42.0 | 2692 |  |
| 125 GR. SIE SP | H4895 | .308" | 2.800" | 41.0 | 2639 |  | 43.0 | 2762 |  |
| 150 GR. HDY SP | H4831 | .308" | 2.790" | 47.0 | 2493 |  | 51.0 | 2659 |  |
| 150 GR. HDY SP | H4350 | .308" | 2.790" | 45.0 | 2420 |  | 49.0 | 2610 |  |
| 150 GR. HDY SP | H414 | .308" | 2.790" | 44.0 | 2529 |  | 46.0 | 2689 |  |
| 150 GR. HDY SP | H380 | .308" | 2.790" | 42.0 | 2536 |  | 44.0 | 2661 |  |
| 150 GR. HDY SP | BL-C(2) | .308" | 2.790" | 40.0 | 2450 |  | 42.0 | 2559 |  |
| 150 GR. HDY SP | H335 | .308" | 2.790" | 40.0 | 2442 |  | 42.0 | 2541 |  |
| 150 GR. HDY SP | H4895 | .308" | 2.790" | 40.0 | 2484 |  | 42.0 | 2578 |  |
| 168 GR. SIE HPBT | H4831 | .308" | 3.060" | 45.0 | 2477 |  | 49.0 | 2578 |  |
| 168 GR. SIE HPBT | H4350 | .308" | 3.060" | 44.0 | 2379 |  | 48.0 | 2524 |  |
| 168 GR. SIE HPBT | H414 | .308" | 3.060" | 42.0 | 2429 |  | 44.0 | 2519 |  |
| 168 GR. SIE HPBT | H380 | .308" | 3.060" | 41.0 | 2414 |  | 43.0 | 2502 |  |
| 168 GR. SIE HPBT | BL-C(2) | .308" | 3.060" | 39.0 | 2363 |  | 41.0 | 2404 |  |
| 168 GR. SIE HPBT | H335 | .308" | 3.060" | 39.0 | 2354 |  | 41.0 | 2399 |  |
| 168 GR. SIE HPBT | H4895 | .308" | 3.060" | 39.0 | 2388 |  | 41.0 | 2432 |  |
| 180 GR. HDY SP | H4831 | .308" | 2.900" | 43.0 | 2304 |  | 47.0 | 2424 |  |
| 180 GR. HDY SP | H4350 | .308" | 2.900" | 42.0 | 2250 |  | 46.0 | 2566 |  |
| 180 GR. HDY SP | H414 | .308" | 2.900" | 40.0 | 2237 |  | 42.0 | 2355 |  |
| 180 GR. HDY SP | H380 | .308" | 2.900" | 38.0 | 2213 |  | 40.0 | 2323 |  |
| 180 GR. HDY SP | BL-C(2) | .308" | 2.900" | 37.0 | 2189 |  | 39.0 | 2284 |  |
| 180 GR. HDY SP | H335 | .308" | 2.900" | 37.0 | 2184 |  | 39.0 | 2263 |  |
| 180 GR. HDY SP | H4895 | .308" | 2.900" | 37.0 | 2201 |  | 39.0 | 2296 |  |